                     Enquiry Form for Cold rolling mill be
Type of Mill
:


4Hi single stand reversing mil

Type of material to be rolled:
 Low Carbon Steel Medium Carbon Steel                                                                                                                                                                                                                                                                                                                           
                                        

High Carbon Steel Alloy Steel like- 16MnCr5
Incoming coil data:
Strip Width
:    
Minimum: 100mm   Maximum: 600 mm
Material Thickness:    
Minimum: 2.00mm  Maximum: 8.0mm
Coil Weight       :    
Maximum: 12 MT



Coil ID

:
508mm/610mm



Coil OD
:
2000mm

Outgoing coil data:
Strip Width
:    
Minimum: 100mm   Maximum: 600 mm

Material Thickness:    
Minimum: 0.50mm  Maximum: 7.0mm

Coil Weight       :    
Maximum: 12 MT



Coil ID

:
508mm



Coil OD
:
2000mm
 UTS of material:
Minimum: 350MPa 
Maximum: 1100MPa
 Yield Strength:
Minimum: 230Mpa
 Maximum: 650MPa
Production Requirements:
Annual Production:
Minimum 60,000MT @85% utilization
Estimated Product Mix :
	
	

	Grade
	I/P THICK (HR COIL)
	Mt/Annum

	16MNCR5
	6MM TO 8.0MM
	10000

	C45 /C62
	5MM TO 8.0MM
	10000

	C30 / C40
	2MM TO 4MM
	5000

	FE410/SAPH440/FE490
	4MM TO 6MM
	5000

	D/513D/DD
	4.5MM
	15000

	
	2MM TO 3MM
	0

	SAE1010
	6MM
	15000

	 
	 
	60000


Line Speed:





Roll Separating Force:

  To be decided by supplier
Maximum Reel tension

Maximum Payoff Tension

Roll Size: (Diameter, Barrel length, and Hardness)
Bearing Size, Type and its lubrication:
    

 To be decided by supplier
Electrical and Automation: AC motor and drives will be preferred

Thickness Gauge:
Non-contact Isotope type or X ray, to be installed both side

Thickness Control:
Automatic Hydraulic AGC with auto tension control. Component of AGC

 control should be feed forward, feed backward and mass flow       
Control Desk:
Entry, Delivery, POR and Pulpit

HMI PC:
Mill and AGC               
Automation   level: 
Latest

