                     Enquiry Form for Pickling Line be
Material to be processed:-
Type of material          :   Low Carbon Steel Medium Carbon Steel                                                                                                                                                                                                                                                                                                                           
                                        High Carbon Steel Alloy Steel like- 16MnCr5
Material Width           :    Minimum: 100mm       Maximum: 600 mm
Material Thickness     :    Minimum: 2.00mm 
 Maximum: 8.0mm
Coil Weight                :    Maximum: 12 MT
UTS                            :    Minimum: 350MPa 
Maximum: 1100MPa
Yield Strength            :    Minimum: 230Mpa
 Maximum: 650MPa
Production Requirements and Working hours:- 

Annual Production         :       ____60,000MT_______ in _______@85% utilization
Estimated Product Mix :
	HR COIL PRODUCT MIX AT HRS
	

	Grade
	I/P THICK (HR COIL)
	Mt/Annum

	16MNCR5
	6MM TO 8.0MM
	10000

	C45 /C62
	5MM TO 8.0MM
	10000

	C30 / C40
	2MM TO 4MM
	5000

	FE410/SAPH440/FE490
	4MM TO 6MM
	5000

	D/513D/DD
	4.5MM
	15000

	
	2MM TO 3MM
	0

	SAE1010
	6MM
	15000

	 
	 
	60000


Strip Edge Trimming?              :     No                                                                             
Electrical Automation              :   Level 2 Reporting    (Level 2)  
Electrical Voltage                    :     440Volts AC      50 Hz                                                                                      

Acid fumes scrubbing – Double stage.
Spent Acid & Rinse water - Effluent Treatment Plant      
Line Speed:
To be decided by supplier

